Part A: Questions

- What is the definition of reactor Start-up Rate (SUR) and its unit?

- A reactor has effective multiplication of 1.0025, what is the stable reactor period and SUR?

- What is the physical meaning of the following parameters:

- Explain Why in a core cycle, effective delayed neutrons () decreases with time (or burn up)?

- What is the mean energy of delayed neutrons and what is the difference of  and β?

- Explain the following figure (each box, left to right and up to down) completely

- Graphically explain In-hour equation in the positive and negative reactivity cases and discuss about the roots

- Classify time dependent problems in the dynamics problems with an example to describe Long-time, Intermediate-Time, and short-time problems.

- Why in a normal operation reactivity should be less than β?

- What are the reactor feedbacks on the reactivity (fuel Temp., coolant Temp., xenon effect, void coeff.)? Discuss each feedback, completely.

- Explain under-moderated and over-moderated reactor with a graph, and also why under-moderation makes self-regulation (with an example)

- What is the reactor dead-time? (b) Based on the following power profile, draw the xenon concentration.

How we can find power from inverse engineering?

- Why  a drop rod accident makes a serious concern in a normal operating condition and in the case of  ? Explain its scenario.

- Explain PWR startup completely using P-T diagram and explain seven major points. Explain about stresses and constraints. (b) What is three mechanism for pressure changing in a PWR?

Part B: Problems

- Estimate neutron life-time in a finite graphite. Use absorption cross section , density=1.6g/cm3, atomic weight=12, thermal neutron speed of 2200m/s, leakage probability=0.89.

- Prove that in the prompt-jump approximation: where  is being initial condition concentration of precursor. (b) From the above formula, what is the reactor period in this case?

- Convert value of reactivity listed below to the indicated units.

- 0.000421=  ……   pcm

- 0.065 =….. mβ

- Find effective multiplication factor when reactivity is equal to 90pcm.

- Derive Two-point kinetic equations using non-steady one-group diffusion equation. (b) derive In-hour equation based on six-group of delayed neutron.

- Suppose a reactivity of 0.2β  is inserted as step change and assume the initial power to be 90%. Find reactor power 5 sec after step changing?

(b) The prompt neutron life time in a thermal reactor is 5.7×10−4s. For reactivity of 0.00065, express the reactor period in terms of second and in-hours. (c) Express reactivity in terms of dollars units.

- The reactivity is suddenly changed to 0.0032. Assuming one delayed neutron, determine change of neutron flux with time and plot those. What is the period?

- Assume matrix representation of points kinetics equation, based on the following formula:

Let ρ = −0.6Δk/k . Estimate the neutron flux function according to one-group delayed neutron approximation. (b) find its period (c)  Explain about photon flux after shut-down. (d) Discuss six-group behavior on neutron and photon flux versus time at the shut-down.

- In a normal operation, load (power and turbine demand) is reduced from 100% to 40% for an under-moderated PWR based on Fig. 1.

Fig. 1: load reduction is shown.PWR:power,PWNT:neutron flux, TBLD:turbine demand.

Now explain the following graph’s behavior, completely.

- When a reactor is critical on prompt neutron alone, it is said to be Upromptcritical.U Show that a reactor with U-235 fuel becomes prompt critical when ρ =β. (20 points)

- A Ramp reactivity insertion such as the following figure is inserted. Calculate the period and explain power profiles in each time interval. (b) Sketch the inverse of period and power as a function of time. (40 points)