Instructions for PRIS data reporting

Part 1: Data structure

PRIS data are reported in five parts:

I) Production Data for calculating the following nuclear power plant performance indicators on monthly and annual basis:

Operation Factor (OF) and Load Factor (LF).

II) Unavailability Data for calculating the following nuclear power plant performance indicators on monthly and annual basis:

Energy Availability Factor (EAF), Planned Energy Unavailability Factor (PUF), Unplanned Energy Unavailability Factor (UUF), Unit Capability Factor (UCF), Unplanned Capability Loss Factor (UCLF), Forced Loss Rate (FLR)

III) Operating Experience Data for brief information on the general performance and operational mode of the plant over the year and significant factors affecting energy generation over the year. Here information is also collected to calculate the Unplanned Automatic and Unplanned Manual Scram Indicators (UA7 and UM7).

IV) Outage Data to look at the plant operation history, to make various outage analyses, etc.

The record is given for all outages of the unit during the year containing the date and duration of the outage, total energy lost, type and cause of the outage and the system involved.

V) Non-Electrical Application Production Data to evaluate the amount of power delivered by the plant for non-electrical applications in the form of heat/steam.

###### PART 2: REPORTING INSTRUCTIONS

I.	Production Data

### Year

Enter the year of operation in the form of yyyy.

### IAEA plant unit code

Enter the PRIS reactor code.

Station name and unit number

Enter the station name and unit number, which of data is reported.

Reference Unit Power (net) [MWe]

Enter the reference unit power (net), expressed in units of megawatt (electrical).

The reference unit power is the maximum (electrical) power that could be maintained continuously throughout a prolonged period of operation under reference ambient conditions. The power value is measured at the unit outlet terminals, i.e. after deducting the power taken by unit auxiliaries and the losses in the transformers that are considered integral parts of the unit.

The reference unit power value should include also the electrical equivalent of the portion of energy delivered in the form of steam/heat that might have been used for non-electrical applications. However, this applies only to the units, where the heat production may reduce the unit electrical power below its maximum value.

If a maximum power capability has been determined by formal test, the reference unit power is determined by correcting test results to reference ambient conditions. If a formal test has not been performed, the reference power should be based on design values, adjusted for reference ambient conditions. The reference unit power is expected to remain constant unless following design changes, or a new permanent authorization, the management decides to amend the original value. (It is recognized that the reference unit power may be based upon an authorized maximum unit thermal power, and in these cases, the "reference unit power (net)" corresponding to the authorized maximum unit thermal power should be used for simplicity in the calculations.)

### Reference Unit Power Revisions and Dates of Revisions

Enter any changes in the reference unit power (net) that are foreseen to be permanent and which occurred during the year. Note that regulatory limitations for the net reference unit power of a non-permanent nature should not be reported here but as a partial outage due to regulatory limitation. (Direct co-user of Outage, Code H, Part IV)

Does your plant supply heat for non-electrical applications?

Please tick the appropriate box according to the actual conditions at your plant. If the answer is yes, i.e. your plant produces a part of its power in form of heat (supplies steam for non-electrical applications to off-site consumers), please report also the amount of thermal energy using the Heat Production Data Sheet (Section V) of this Questionnaire. However, this applies only to those plants, where the heat production may reduce the unit electrical power below the reference unit power.

Monthly Energy Generation (net) during the year,

### Energy Generated (net), EG [MWeh]

Enter the net electrical energy produced during the reference period as measured at the unit outlet terminals, i.e. after deducting the electrical energy taken by unit auxiliaries and the losses in transformers that are considered integral parts of the unit.

On-line Hours, t [hours]

Enter the total clock hours in the month during which the unit operated with at least one main generator connected to the grid.

Reference Period, T [hours]

Generally, enter the total number of hours in the calendar month.

For units being commissioned during the month, enter the clock hours from the beginning of the month or the first connection to the grid, whichever comes last, to the end of the month.

For units being in commercial operation at the beginning of the month, enter the clock hours from the beginning of the month to the end of the month or to the last disconnection from the grid before permanent shutdown, whichever comes first.

### Lifetime Cumulative Energy Generation (net) [GWeh]

Enter net electricity generated since the first connection to the grid. For its actual value, add the total EG for the current reporting period to the last reported Lifetime Cumulative Electricity Generation value.

II.	Unavailability Data

For the purpose of the PRIS outage coding system, the unit unavailability is defined as a status of power  plant unit, when it is not able to operate at its reference power for technical or environmental reasons. This condition, which may be under or beyond plant management control, reflects only lack of power production capability of the unit itself. It does not include situations, when a power plant unit was operated at  a reduced power, while it would be able to operate at the reference unit power (for example due to low energy demand, transmission grid failures or unfavourable political situation in the country). It follows from the above definition that unit operation at reduced power does not always imply unit unavailability. In other words, a unit may be operated at a reduced power, even though it is fully available.

In the Unit Unavailability Data form, enter only energy losses, which have been caused by power plant unit unavailability as defined above. Unit power reduction due to supply of energy to a nearby unit (e.g. in the form auxiliary steam or auxiliary electrical power in case of the nearby unit startup) is considered a unit unavailability under plant management control. Energy losses due to these reasons must be entered in the Unavailability Data Form.

Do not enter energy losses due to reasons like a grid failure, load following operation, or government/court decisions, when the unit was fully available (capable of reaching the reference unit power). For completeness of information on lost energy due to various reasons, however, the energy losses with the unit fully available should be reported in the Outage Data form. (See the outage cause codes (J), (K), (M), (R), (S), and (U) in the Chapter IV of the Reporting Instructions).

The calculation of energy losses due to reduced unit availability is always related to the reference unit power.

Three types of energy losses caused by unit unavailability have been defined in PRIS: planned energy losses, unplanned energy losses, which both are due to causes controllable by the plant management,  and other energy losses, which are due to constraints that the plant management cannot control.

Planned Energy Losses (net), PEL [MWeh]

Enter the energy, expressed in megawatt-hours (electric), that was not delivered during the month because of planned shutdowns or planned load reductions due to causes under the plant management control.

The shutdowns or load reductions are considered planned, if they are scheduled at least four weeks in advance (generally at the time when the annual overhaul, refuelling or maintenance programme is established), and if the beginning of the unavailability period can be largely controlled and deferred by plant management.

Unplanned Energy Losses (net), UEL [MWeh]

Enter the energy, expressed in megawatt-hours (electric), which was not delivered during the month because of unplanned shutdowns, unplanned load reductions or outage extensions due to causes under the plant management control.

The shutdowns or load reductions due to causes under plant management control are considered unplanned, if they are not scheduled at least four weeks in advance.

When evaluating UEL, the unit is supposed to be running under reference ambient conditions.

Forced energy losses FEL [(MWeh)]

In this column, enter the energy, expressed in megawatt-hours (electric), that was not delivered during the month because of unplanned shutdowns or unplanned load reductions due to causes under the plant management control.

Extensions of planned energy losses EPL [(MWeh)]

In this column, enter the energy, expressed in megawatt-hours (electric), that was not delivered during the month due to unplanned extensions of planned load reductions or shutdowns, if causes of these extensions were under the plant management control.

Additional information on planned and unplanned energy losses is provided in the chapter IV, “Outage Data (Type code)” and in the Part 3: “Definitions of Performance Indicators”.

### Other Energy Losses (net), XEL [(MWeh)]

Enter the energy, expressed in megawatt-hours (electric), that was not delivered during the month due to constraints reducing plant availability and being beyond the plant management control.

Energy losses caused by the following conditions should be reported here,

- Environmental conditions (seasonal variations in cooling water temperature, flood, storm, lightning, lack of cooling water due to drought, tidal valves, high sea or water intake restrictions that could not be prevented by operator action)

- Fuel coastdown (power reduction at the end of fuel cycle resulting in release of a positive reactivity to compensate for high fuel burn-up)

- Restrictions on supply and services due to external constraints (lack of funds due to delayed payments from customers, disputes in fuel industries, fuel-rationing, labour strike outside the plant, spare part procurement difficulties etc.)

- III.	Operating Experience Data

- Highlights of Operation

- State briefly the general performance and operational mode of the plant over the reporting period e.g.

- Operation at full power in base load mode

- Load-following for a period

- Shut-down for a period

- Major achievements leading to increased availability

- Describe the significant factors affecting energy generation over the reporting period, e.g.

- Limitations introduced by regulatory bodies

- Limitations due to fuel management

- Shortage of consumables

- Personnel factors

- Equipment performance

- Environmental conditions

- Number of critical hours in the reporting period:

- Enter the number of hours during the reporting period when the reactor was critical.

- Number of unplanned automatic scrams in the reporting period:

- Enter the number of unplanned automatic scrams that occurred during the reporting period while the reactor was critical.

- Number of unplanned manual scrams in the reporting period:

- Enter the number of unplanned manual scrams that occurred during the reporting period while the reactor was critical.

- IV.	Outage Data

- For the purpose of PRIS coding, the outage is defined as any status of a reactor unit, when its actual output power is lower than the reference unit power for a period of time. By this definition, the outage includes both power reduction and unit shutdown, however it is recognised that in a common understanding it may mean the shutdown only.

- Report all significant outages including outage extensions and reactor scrams. The outage is considered significant, if the loss in the energy production corresponds to at least ten hours of continuous operation at the reference unit power or if it has been caused by an unplanned reactor scram (even if the unit had been shut down for less than 10 hours). It is desirable to report also smaller than significant outages. If more outages occurred at a time, they would be considered as separate outages and reported as if the unit was operating at the reference power.

- The reactor scram is defined as a reactor shutdown achieved by rapid insertion of negative reactivity into the reactor core, which can be performed either manually or automatically. All unplanned reactor scrams must be reported, even if they occurred after the unit was disconnected from grid (when the reactor remained at power). Planned scrams performed as a part of planned tests are not reported.

- The outage extension is defined as the unplanned portion of a planned outage, causing prolongation of the planned outage beyond its originally planned completion date. Outage extension must be always reported as unplanned, unless it is announced at least four weeks in advance. The planned outage extensions are considered a part of the planned outages and are not reported separately.

- Start Date

- Enter the first day of the outage in the form of “yyyymmdd”, e.g. 20001228 for December 28, 2000.

- If no start date can be specified (e.g. for a continuous load following operation), enter the first day of the reporting period. The same applies if an outage extends from the previous reporting period.

### Duration [hours]

- Enter the total time of the outage measured in full clock hours (rounded) from the beginning of the reporting period or the outage, whichever comes last, to the end of the reporting period or the outage, whichever comes first. The time includes both the power decrease and power rise period.

- If a part of the outage extends to the next reporting period, the corresponding outage duration is coded for each reporting period separately. For intermittent outages (e.g. due to load following operation), enter cumulative data for the reporting period. For reactor scrams after disconnection of the unit from the grid, no outage duration is reported (the field is left blank).

- Energy Loss (net) [(MWeh)]

- Enter the total energy expressed in megawatt-hours (electric) that has not been delivered to the grid or other consumers due to the outage.

- Energy losses are calculated separately for each outage. If several outages are concurrent for a period of time, energy loss for each outage is reported as if the unit was operated at the reference power at the beginning of the outage. For reactor scrams after disconnection of the unit from the grid, no energy loss is reported (the field is left blank).

- For intermittent outages (e.g. due to load following operation), enter cumulative data for the reporting period.

- Type code

- Enter a two- or three-character code for the outage type. The individual outage types will be coded as follows:

- First character:

| # Code | - Description |
| --- | --- |
| - P | - Planned outage due to causes under the plant management control |
| - U | - Unplanned outage due to causes under the plant management control |
| - X | - Outage due to causes beyond the plant management control ("external") |

- An outage is considered planned, if it has been scheduled at least four weeks in advance.

- An outage is considered unplanned, if it has not been scheduled at least four weeks in advance.

- The “external” outages may be also considered planned or unplanned. Although this aspect is not explicitly coded, adding the third character (see below) to the "external" outage code will imply the unplanned “external” outage.

- In general, any change in the planned outage start date is considered unplanned, unless it is announced at least four weeks in advance. If the start date is anticipated, the outage is considered unplanned until the originally scheduled start date. If the start date is postponed, the outage is still considered planned until the originally scheduled completion date. Any extension of the planned outage beyond the original completion date is considered unplanned, unless it is announced at least four weeks in advance. The unplanned portions of planned outages due to changes in outage start date should be coded as separate outages.

Exceptions from this rule are provided in the Part 3, Detailed Descriptions of PRIS Performance Indicators.

Second character:

| # Code | - Description |
| --- | --- |
| F | Full outage |
| P | Partial outage |

An outage is considered full if the actual unit output power has been reduced to zero percent (unit disconnected from all off-site power supply lines). An outage is considered partial if the actual unit output power is lower than its reference value, but is greater than zero percent.

Third character (for unplanned outages only):

| # Code | - Description |
| --- | --- |
| 1 | Controlled shutdown or load reduction that could be deferred but had to be performed earlier than four weeks after the cause occurred or before the next refuelling outage, whatever comes first |
| 2 | Controlled shutdown or load reduction that had to be performed in the next 24 hours after the cause occurred |
| 3 | Extension of planned outage |
| 4 | Reactor scram, automatic |
| 5 | Reactor scram, manual. |

The third character should be assigned also to outages due to causes beyond plant management control ("external"), which can be considered unplanned (see the cause codes J, M, N, R, T and U below).

Thus the outage type may have one of the following codes:

PF		or	PP

UF1-5	or	UP1-3

XF		or	XP

XF1-5	or	XP1-3

Cause code

At each outage type, enter one- to five-character code describing the outage cause and the system primarily involved or affected in the outage.

First character represents a direct cause of the outage.

The direct cause is defined as an immediate action or condition that has directly resulted in the outage. For example, if a minor equipment failure, as an oil leak dropping on a hot pipeline or a short-circuit in a non-vital switchgear cabinet, results in an extensive fire that directly causes an outage, the fire is considered the direct cause of the outage.

For a particular outage (full or partial), only one cause may be selected.

If outages occur successively, they must be reported as separate outage due to different causes.  For example, if unit power was first reduced due to an equipment failure, but the unit subsequently tripped due to a human error when responding to the failure, these incidents should be reported as two separate outages caused by equipment failure and human factor respectively. Similarly, partial and full outage following immediately one upon the other and having the same direct cause must be reported separately.

In the first character position, enter one letter from the below table of outage direct causes.

### Direct causes of outages

| # Code | - Description |
| --- | --- |
| A | Plant equipment failure |
| B | Refuelling without a maintenance |
| C | Inspection, maintenance or repair combined with refuelling |
| D | Inspection, maintenance or repair without refuelling |
| E | Testing of plant systems or components |
| F | Major back-fitting, refurbishment or upgrading activities with refuelling |
| G | Major back-fitting, refurbishment or upgrading activities without refuelling |
| H | Nuclear regulatory requirements |
| J | Grid failure or grid unavailability |
| K | Load-following (frequency control, reserve shutdown due to reduced energy demand) |
| L | Human factor related |
| M | Governmental requirements or Court decisions |
| N | Environmental conditions (flood, storm, lightning, lack of cooling water due to dry weather, cooling water temperature limits etc.) |
| P | Fire |
| R | External restrictions on supply and services (lack of funds due to delayed payments from customers, disputes in fuel industries, fuel-rationing, labour strike outside the plant, spare part delivery problems etc.) |
| S | Fuel management limitation (including high flux tilt, stretch out or coast-down operation) |
| T | Heat supply (on-site to support next unit or desalination and off-site distribution) |
| U | Security and access control |
| Z | Others |

Planned outages may be due to causes coded B, C, D, E, F, G; unplanned outages may be due to causes coded A, H, L, P; "external" outages may be due to causes coded J, K, M, N, R, T and U. The cause coded S can apply to planned, unplanned and "external" outages. Outages due to energy supply to a nearby unit in a form of steam should be coded T.

Causes related to equipment (A), repair (D), testing (E), back-fitting (F, G), nuclear regulatory requirements (H), human actions (L), environmental conditions (N), fire (P), fuel management (S) and other (Z) should, whenever applicable, be followed by the numerical code of the plant system affected by or otherwise involved in the outage .

The second to fifth characters represent the plant system primarily involved in /affected bythe outage.

In the second to fifth- character positions, enter a code of the particular system. Choose a system code from the below table.

For a single outage (full or partial), only one system may be selected. If two or more systems were involved in the outage, select either the system directly causing the outage or the one being most significantly affected.

If no particular system could be specified from the general system group, enter the general system code “xx.00”

In case a particular system was involved in the outage, but no suitable code was found in the list, choose the appropriate general system group and enter the “other” code ”xx.99”.

If no system was involved/affected in the outage, leave blank the second to fifth characters in the outage cause code.

Plant Systems Possibly Involved in the Outage

| ## Code | ## System Description |
| --- | --- |
| Nuclear Systems |  |
| 11.00 | Reactor and Accessories |
| 11.01 | Reactor vessel and main shielding (including penetrations and nozzles) |
| 11.02 | Reactor core (including fuel assemblies) |
| 11.03 | Reactor internals (including steam separators/dryers - BWR, graphite, pressure tubes) |
| 11.04 | Auxiliary shielding and heat insulation |
| 11.05 | Moderator and auxiliaries (PHWR) |
| 11.06 | Annulus gas system (PHWR/RBMK) |
| 11.99 | None of the above systems |
| 12.00 | Reactor I&C Systems |
| 12.01 | Control and safety rods (including drives and special power supply) |
| 12.02 | Neutron monitoring (in-core and ex-core) |
| 12.03 | Reactor instrumentation (except neutron) |
| 12.04 | Reactor control system |
| 12.05 | Reactor protection system |
| 12.06 | Process computer |
| 12.07 | Reactor recirculation control (BWR) |
| 12.99 | None of the above systems |
| 13.00 | Reactor Auxiliary Systems |
| 13.01 | Primary coolant treatment and clean-up system |
| 13.02 | Chemical and volume control system |
| 13.03 | Residual heat removal system (including heat exchangers) |
| 13.04 | Component cooling system |
| 13.05 | Gaseous, liquid and solid radwaste treatment systems |
| 13.06 | Nuclear building ventilation and containment inerting system |
| 13.07 | Nuclear equipment venting and drainage system (including room floor drainage) |
| 13.08 | Borated or refuelling water storage system |
| 13.09 | CO2 injection and storage system (GCR) |
| 13.10 | Sodium heating system (FBR) |
| 13.11 | Primary pump oil system (including RCP or make-up pump oil) |
| 13.12 | D2O leakage collection and dryer system (PHWR) |
| 13.13 | Essential auxiliary systems (GCR) |
| 13.99 | None of the above systems |
| 14.00 | Safety Systems |
| 14.01 | Emergency core cooling systems (including accumulators and core spray system) |
| 14.02 | High pressure safety injection and emergency poisoning system |
| 14.03 | Auxiliary and emergency feedwater system |
| 14.04 | Containment spray system (active) |
| 14.05 | Containment pressure suppression system (passive) |
| 14.06 | Containment isolation system (isolation valves, doors, locks and penetrations) |
| 14.07 | Containment structures |
| 14.08 | Fire protection system |
| 14.99 | None of the above systems |
| 15.00 | Reactor Cooling Systems |
| 15.01 | Reactor coolant pumps/blowers and drives |
| 15.02 | Reactor coolant piping (including associated valves) |
| 15.03 | Reactor coolant safety and relief valves (including relief tank) |
| 15.04 | Reactor coolant pressure control system |
| 15.05 | Main steam piping and isolation valves (BWR) |
| 15.99 | None of the above systems |
| 16.00 | Steam generation systems |
| 16.01 | Steam generator (PWR), boiler (PHWR, AGR), steam drum vessel (RBMK, BWR) |
| 16.02 | Steam generator blowdown system |
| 16.03 | Steam drum level control system (RBMK, BWR) |
| 16.99 | None of the above systems |
| 17.00 | Safety I&C Systems (excluding reactor I&C) |
| 17.01 | Engineered safeguard feature actuation system |
| 17.02 | Fire detection system |
| 17.03 | Containment isolation function |
| 17.04 | Main steam/feedwater isolation function |
| 17.05 | Main steam pressure emergency control system (turbine bypass and steam dump valve control) |
| 17.06 | Failed fuel detection system (DN monitoring system for PHWR) |
| 17.07 | RCS integrity monitoring system (RBMK) |
| 17.99 | None of the above systems |
| Fuel and Refuelling Systems |  |
| 21.00 | Fuel Handling and Storage Facilities |
| 21.01 | On-power refuelling machine |
| 21.02 | Fuel transfer system |
| 21.03 | Storage facilities, including treatment plant and final loading and cask handling facilities |
| 21.99 | None of the above systems |
| Secondary plant systems |  |
| 31.00 | Turbine and auxiliaries |
| 31.01 | Turbine |
| 31.02 | Moisture separator and reheater |
| 31.03 | Turbine control valves and stop valves |
| 31.04 | Main condenser (including vacuum system) |
| 31.05 | Turbine by-pass valves |
| 31.06 | Turbine auxiliaries (lubricating oil, gland steam, steam extraction) |
| 31.07 | Turbine control and protection system |
| 31.99 | None of the above systems |
| 32.00 | Feedwater and Main Steam System |
| 32.01 | Main steam piping and valves |
| 32.02 | Main steam safety and relief valves |
| 32.03 | Feedwater system (including feedwater tank, piping, pumps and heaters) |
| 32.04 | Condensate system (including condensate pumps, piping and heaters) |
| 32.05 | Condensate treatment system |
| 32.99 | None of the above systems |
| 33.00 | Circulating Water System |
| 33.01 | Circulating water system (pumps and piping/ducts excluding heat sink system) |
| 33.02 | Cooling towers / heat sink system |
| 33.03 | Emergency ultimate heat sink system |
| 33.99 | None of the above systems |
| 34.00 | Miscellaneous Systems |
| 34.01 | Compressed air (essential and non-essential / high-pressure and low-pressure) |
| 34.02 | Gas storage, supply and cleanup systems (nitrogen, hydrogen, carbon dioxide etc.) |
| 34.03 | Service water / process water supply system (including water treatment) |
| 34.04 | Demineralized water supply system (including water treatment) |
| 34.05 | Auxiliary steam supply system (including boilers and pressure control equipment) |
| 34.06 | Non-nuclear area ventilation (including main control room) |
| 34.07 | Chilled water supply system |
| 34.08 | Chemical additive injection and makeup systems |
| 34.09 | Non-nuclear equipment venting and drainage system |
| 34.10 | Communication system |
| 34.99 | None of the above systems |
| 35.00 | All other I&C Systems |
| 35.01 | Plant process monitoring systems (excluding process computer) |
| 35.02 | Leak monitoring systems |
| 35.03 | Alarm annunciation system |
| 35.04 | Plant radiation monitoring system |
| 35.05 | Plant process control systems |
| 35.99 | None of the above systems |
| Electrical Systems |  |
| 41.00 | Main Generator Systems |
| 41.01 | Generator and exciter (including generator output breaker) |
| 41.02 | Sealing oil system |
| 41.03 | Rotor cooling gas system |
| 41.04 | Stator cooling water system |
| 41.05 | Main generator control and protection system |
| 41.99 | None of the above systems |
| 42.00 | Electrical Power Supply Systems |
| 42.01 | Main transformers |
| 42.02 | Unit self-consumption transformers (station, auxiliary, house reserve etc.) |
| 42.03 | Vital AC and DC plant power supply systems (medium and low voltage) |
| 42.04 | Non-vital AC plant power supply system (medium and low voltage) |
| 42.05 | Emergency power generation system (e.g. emergency diesel generator and auxiliaries) |
| 42.06 | Power supply system logics (including load shed logic, emergency bus transfer logic, load sequencer logic, breaker trip logic etc.) |
| 42.07 | Plant switchyard equipment |
| 42.99 | None of the above systems |

### Description of the outage

Describe briefly the direct cause of the outage, the operational mode of the plant at the time of the outage occurrence and specify the systems involved including their components. This field should provide at least a name describing nature of the outage.

In the cause description, specify in more details the general causes: the type of human factor (operator mistake, omission, failure to monitor plant processes), the type of equipment failure (spurious actuation of a system, component trip, damage or malfunction), the type of adverse environmental condition (frost, lightning, high sea), the type of load following operation (frequency control, reserve shutdown), the cause of fire etc. If applicable, specify also the cause coded Z – Others, the system involved coded “xx.99” or other major systems involved, if not coded.

For the operational mode description, choose one of the following operational modes describing the status of the unit immediately before the outage.

- Power operation

- Start-up/shutdown operation

- Hot standby (reactor subcritical)

- Hot shutdown (reactor subcritical)

- Cold shutdown (reactor subcritical)

- Reactor pressure vessel open

If applicable, please provide in more details the actual type of operation/activity in the particular mode (e.g. power ascension after an outage; steady power operation at rated or reduced power upon the grid dispatcher’s request etc.).

- V.	Non-electrical Application Production Data

Some power plant units produce a portion of their output energy in the form of heat/steam for non-electrical applications (desalination, district heating and industrial heat). This energy should be also reported, provided the production of heat/steam reduces the actual output electrical power below the reference unit power as defined in Section I.

Monthly energy provision during the year to “off-site” heat application systems:

Thermal Energy Provided, PDH, PPH, PDS, PDH+PPH+PDS [Gcal]

Enter the thermal energy provided during the reference period to the “off-site” heat application systems (district heating, industrial process heat delivery or/and distillation-type desalination) in the form of heat (steam or hot water) as measured as the difference between the plant feed (outlet) and return (inlet) headers.

If it is difficult to provide individual data for (PDH), (PPF) and (PDS), the total delivery data for (PDH+PPH+PDS) are acceptable. If monthly data are difficult to provide, the annual total data are acceptable.

### Equivalent energy generated for “off-site” non-electrical application during the reporting period, NE1 [MWeh]

Enter the sum of electrical equivalent of energy supplied to “off-site” non-electrical application systems during the reporting period in the form of steam expressed in megawatts-hours (electric). It should be equal to the sum of heat energy delivered to heat application systems (“TOTAL” PDH+PPH+PDS in V.1).

To calculate equivalent energy, the following formula may be applied.

NE1 [MWeh] = 1.16 x ”TOTAL” (PDH+PPH+PDS) in [Gcal] x “Average” thermal efficiency of the unit

Where the “Average” is the thermal efficiency of the unit over the reporting period. If the “Average” value is not available for each unit, the overall “Average” of the plant or the “Default” value may be used. The “Default” value is 0.30 estimated by the designed capacity of all co-generating plants in the PRIS database. If no value is given here, the PRIS calculates the value using this “Default” value.

Lifetime accumulated equivalent energy generated for non-electrical application, LE1 [MWe·h]

The program calculates sum of NE1 above over the whole life of the unit automatically. When all the historical data are not available, it will be so mentioned.

Back-up Source

A non-nuclear back-up heat source may be installed at a nuclear power plant with multiple units in order to secure heat delivery capability from the plant under control of the nuclear power plant in order to compensate insufficient availability of heat from the nuclear reactor.

Monthly energy delivery to, and water production of, “on-site” desalination systems during the year:

Thermal Energy Provided for Distillation, PDI [Gcal]

Enter the thermal energy provided during the reference period to the desalination systems of distillation type (multi-stage flash or multi-effect distillation) or in the form of heat as measured as the difference between at the plant feed (outlet) and return (inlet) headers, or between at the heat extraction points and return points in the case of in-plant facilities.

If it is difficult to provide monthly data, the annual total data are acceptable.

Electrical Energy Provided for Reverse Osmosis Process, PRO [MW(e)·h]

Enter the electrical energy provided during the reference period to the desalination systems of reverse osmosis process. By definition this electrical energy is a part of in-site power.

If it is difficult to provide monthly data, the annual total data are acceptable.

Water Production, WPR [m3], and its Fraction delivered to “off-site” consumers, WDL [m3].

Enter the water production and its fraction delivered to “off-site” consumers during the reference period by the desalination systems.

If it is difficult to provide monthly data, the annual total data are acceptable.

### Equivalent energy consumed by “on-site” desalination for “off-site” delivery during the reporting period, NE2 [MWeh]

Enter the sum of electrical equivalent of energy used for desalination for “off-site” delivery (Fraction of “TOTAL” PDI +PRO in V.2 for “off-site” delivery). The equivalent energy is calculated on the assumption that the fraction of energy consumed for “off-site” delivered water is equal to the fraction of the water delivered to “off-site”.

To calculate equivalent energy, the following formula may be applied.

NE2 [MWeh] = 1.16 x ”TOTAL” (PDI) in [Gcal] x “Average” thermal efficiency of the unit x TOTAL [WDL] /TOTAL [WPR]+TOTAL [PRO] in [MW(e)h] x TOTAL [WDL] /TOTAL [WPR]

Where the “Average” is the thermal efficiency of the unit over the reporting period. If the “Average” value is not available for each unit, the overall “Average” of the plant or the “Default” value may be used. The “Default” value is 0.30 estimated by the designed capacity of all co-generating plants in the PRIS database. If no value is given here, the PRIS calculates the value using this “Default” value.

Lifetime accumulated equivalent energy generated for non-electrical application, LE2 [MWe·h]

The program calculates sum of NE2 above over the whole life of the unit automatically. When all the historical data are not available, it will be so mentioned.

### Equivalent total energy generated for non-electrical application during the reporting period,

### NEG [MWeh]

The program calculates sum of NE1 and NE2 as = NE1 [MWeh] + NE2 [MWeh].

Lifetime accumulated equivalent energy generated for non-electrical application, LEG [MWe·h]

The program calculates sum of LE1 and LE2 above over the whole life of the unit automatically. When all the historical data are not available, it will be so mentioned.

If the operation of non-electric application systems is limited by insufficient heat delivery from the nuclear heat source(s), please explain.

- Give descriptive explanations, in Part IV, what caused this, what countermeasures were taken, etc.

Highlights of energy delivery to non-electric application systems:

Any relevant information in texts may be entered.

Note) At some nuclear power plants (e.g., in Russia), there may be multiple feed/return headers, one each for respective application systems.

- For non-electrical applications

the period of 24 hours

The letters “I”, “O” and “Q” have been deliberately omitted to avoid confusing with digits “0” and “1”

Outages caused by plant personnel strikes should be coded “L”, Human factor related