WANO

July 2005

Error Prevention Tools and Programmes

World Association of Nuclear Operators

GENERAL DISTRIBUTION

Plant Area: Human Performance

Key Words: error prevention, error prevention tools,

operating experience, self-assessments

corrective actions

COPYRIGHT – 2005 World Association of Nuclear Operators (WANO). Not for sale, nor for commercial use. All other rights reserved. This document is protected as an unpublished work under the copyright laws of all countries that are signatories to the Berne Convention and the Universal Copyright Convention. Unauthorised reproduction is a violation of applicable law. Translations are permitted.

LIABILITY DISCLAIMER NOTICE: This information was prepared in connection with work sponsored by WANO. Neither WANO, WANO members, nor any person acting on the behalf of them (a) makes warranty or representation, expressed or implied, with respect to the accuracy, completeness or usefulness of the information contained in this document or that use of any information, apparatus, method or process disclosed in this document may not infringe on privately owned rights or (b) assumes any liabilities with respect to the use of or for damages resulting from the use of any information, apparatus, method or process disclosed in this document.

Introduction

Experience throughout the nuclear industry has repeatedly shown that the likelihood for an error to occur during a plant activity or evolution can be significantly reduced through consistent application of error prevention tools and programmes. This document provides a compilation of error prevention tools commonly used within the nuclear industry that have been successful in preventing errors to occur.

The tools and programmes described in this document are as follows:

Tools

- self-checking

- peer-checking

- concurrent verification

- independent verification

- questioning attitude

- three-way communication

- procedure use and adherence

- procedure place-keeping

- flagging

- pre-job briefings

- post-job debriefs

Programmes

- self-assessments

- use of operating experience

- corrective actions

- observations

Verification tools

Verification tools are used to prevent errors while performing an action or to confirm the correct position of an object after a manipulation. There are four different verification tools— self-checking, peer-checking, concurrent verification and independent verification. These tools can be used by all plant personnel, as well as non-plant personnel.

| self-checking | Self-checking is designed to enhance one’s attention before performing a specific action on plant equipment. Self-checking is a technique in which a person consciously and deliberately reviews the intended action and expected response before performing the action in question. |
| --- | --- |
| peer-checking | Peer-checking is a series of actions by two individuals working together at the same time and place, before and during a specific action, to prevent an error by the performer. |
| concurrent verification | Concurrent verification is a series of actions by two individuals working together at the same time and place to confirm separately the condition of a component before, during and after an action. Concurrent verification is especially useful for activities that involve an action, where an incorrect action could lead to immediate and irreversible harm to the plant. |
| independent verification | Independent verification is a series of actions by two individuals working independently to confirm the condition of a component after the original act that placed it in that condition. |

Source: WANO document ‘Verification Techniques’ – July 2005

Procedure use tools

One way to reduce the likelihood of errors to occur while performing a plant evolution is through the proper use of procedures.

| procedure use and adherence | A plant should be operated in accordance with applicable plant procedures that reflect the plant design bases. Requirements for procedure use should be defined and clearly communicated to plant staff. Commonly seen procedure use levels are as follow: - Continuous use, in which each step of the procedure is read before that step is performed and each step is performed in the sequence given. Typically, each step is then signed off or checked off by the performer. - Reference use, in which the procedure is referred to occasionally during an activity to confirm the correct actions are being performed. Typically, these procedures are in hand or near the job site during the evolution. - Information use, in which an activity may be performed from memory, but the procedure is reviewed prior to the evolution as needed so the operator understands all actions. |
| --- | --- |
| procedure place-keeping | Procedure place-keeping is the use of some device to keep track of which step in the procedure the user is "on" at any given time. Place-keeping is especially important during use of emergency or off-normal procedures, when the stress level is high, the pace may be fast and multiple procedures may be in use at the same time. Place-keeping helps avoid human error by helping to ensure all of the steps of the procedure are completed. |
| flagging | Flagging is a distinct way to identify components to be worked or manipulated, to ensure that staff do not work or manipulate wrong components that are similar in location or appearance. This defence is particularly effective for work on components associated with multiple units or trains of equipment. Flagging is intended to provide an additional barrier so that when an individual is met with a distraction, he or she returns to the right component prior to continuing work. Flagging is not a substitute for a proper self-checking or peer-checking or any verification requirements. |

Source: WANO GL 2001-02, ‘Guidelines for the Conduct of Operations at Nuclear Power Plants’

Pre-job briefings and post-job debriefs

| pre-job briefings | Pre-job briefings are intended to reduce the likelihood of errors during the task. They provide an opportunity for craftsmen, operators and supervisors to preview and discuss important aspects of tasks before starting work. Effective pre-job briefings ensure that personnel have a clear understanding of: - assigned task - roles and responsibilities for the work to be accomplished - critical steps - potential error likely situations - potential consequences of errors - performance expectations necessary to prevent adverse consequences The aim of the brief is to focus team members on the task at hand and to get them thinking proactively, to foresee problems and prevent errors from occurring during the work. |
| --- | --- |
| post-job debriefs | Post-job debriefs are intended to provide learning points that will improve performance of the task the next time it is performed. Post-job debriefs enhance identification and documentation of low level issues that is an extremely important concept for improving future performance. The extent, detail and length of the post-job debrief should be equal to the complexity and significance of the work. Include as many participants of the work as practical to capture lessons learned. Determine what went well and look at near miss issues that can make future work of the same nature safer and more efficient. Identify any problems such as with procedures, planning or equipment and record any issues raised so they can be formally tracked to completion. |

Source: WANO document Conduct of Pre-job Briefings and Post-job Debriefs – March 2004

Other tools

| Three-way communication | Three-way communication creates understanding among individuals involved in the transmittal of key plant information. A sender initiates a message, the receiver acknowledges the message and the sender confirms the accuracy of the message received by the receiver. Senders who initiate the communication are responsible for understanding by both parties. Excellent verbal communication is characterized by: - use of names to get the attention of the receiver (including work areas if using radios) - paraphrased repeat-backs of information (aids understanding) - level of emotion or concern matches the content of the message - word for word repeat backs of equipment nomenclature and plant data - use of phonetic alpha-numeric information |
| --- | --- |
| questioning attitude | Questioning attitude encourages foresight to precede actions, to make sure planning, judgement and decision-making are appropriate for the situation. A questioning attitude promotes awareness of uncertainty and hazards and must overcome the temptation to rationalize away ‘gut feelings’ of something not right. Knowing that it is difficult for workers to see their own mistakes, colleagues, supervisors and even subordinates are encouraged to question, without invitation, another’s action or decision if not understood. This method can be used to confirm the authenticity of a specific action and to validate the appropriateness of the action to achieve desired results. |
| conservative decision-making (stop when unsure) | When an operator, craftsman or technician is faced with unexpected or uncertain conditions he or she should stop the activity and place the equipment or plant in a safe known condition. This include that operators must not hesitate to reduce reactor power or trip the reactor if necessary. The staff subsequently seeks advice from those who possess the proper expertise, such as a supervisor or peer. |

Error prevention programmes

| self-assessments | The objective of a self-assessment programme is to achieve continuous improvement. Current performance is compared to management expectations, worldwide industry standards of excellence and regulatory requirements to identify areas needing improvement. The self-assessment programme should help identify low level issues or trends for early resolution before more significant problems occur that can adversely affect plant safety or reliability. Self-assessments also identify strengths that can be used by other plant groups. The following two different kinds of self-assessments exist: - Ongoing self-assessments—Employees throughout the organisation conduct routinely ongoing self-assessments to verify that standards of performance are being achieved. - Focused self-assessments—Focused self-assessments are performed to evaluate programmes, processes or performance areas against specific criteria. Source: WANO GL 2001-07, ‘Principles for Effective Self-Assessment and Corrective Action Programmes’ |
| --- | --- |
| use of operating experience | The purpose of the use of operating experience is to identify applicable lessons and effectively provide this information to plant personnel, often using diverse methods. These methods include both long-term and short-term training, pre-job briefings, shift briefings, daily meetings and dissemination of information through plant periodicals or company intranet system. Applicable lessons originate from analyses of both internal and external events, as well as other plant programmes. A key to effectively using operating experience is for the right information to be communicated to the right people in time to make a difference. It is unreasonable to expect workers to recall lessons learned from training that was provided months or, perhaps, years earlier. Source: WANO GL 2003-01, ‘Guidelines for Operating Experience at Nuclear Power Plants’ |

| corrective actions | Corrective actions are identified and implemented based on identified deviations from expected standards from different plant programmes, such as self-assessments, management observation programme and soon. In addition, corrective actions are identified based on analyses of both internal and external events. Corrective actions include actions, such as specific inspection or testing of plant equipment, modification of plant equipment or components, changes to preventive maintenance or surveillance testing and changes to operating processes or processes affecting other plant groups. Corrective actions should be implemented in a timely manner. Sources: WANO GL 2001-07, ‘Principles for Effective Self-Assessment and Corrective Action Programmes’ and GL 2003-01, ‘Guidelines for Operating Experience at Nuclear Power Plants’ |
| --- | --- |
| observations | The purpose of an observation programme is to reduce the risk for human errors occurring during different plant activities. The programme is based on observations of plant staff performing activities out in the plant or sometimes in the office. An observation can be performed by a manager, supervisor or a peer. Observations may include the following: - Check work against the work plan. - Assess presence of obstacles to performance. - Identify error-likely situations. - Determine if scope of work has changed. - Verify availability of appropriate tools. - Ask if worker has enough information to complete task. - Check if worker possesses proper skills. - Monitor overall work environment. - Verify if workers accurately perceive risk and priorities. - Reinforce and coach workers on observed behaviours. |