TC Project IRA/2/011/12/01

ORIGINAL: English

INTERNATIONAL ATOMIC ENERGY AGENCY

REPORT OF THE

TECHNICAL COOPERATION Expert Mission:

EXPERT MISSION TO ASSISTS BNPP TO STRENGTHEN ITS SAFETY CAPABILITIY, USING ULTRASONIC TEST METHOD, STANDARDS AND EVALUATION

# REPORT TO THE BUSHEHR NPP

# BUSHEHR, IRAN

6 to 8 July, 2013

EXPERT MISSION

conducted under IAEA Technical Co-operation Project IRA//2/011/12/01
Strengthening Owner’s Functions for BNPP Project

DEPARTMENT OF TECHNICAL CO-OPERATION

Division for Europe, Latin America and West Asia

Organized by: 	IAEA

Technical Co-operation Project IRA//2/011/12/01

Prepared by:		Mato Cvitanovic	Croatia

Gyozo Paczolay	Hungary

CONTENTS

1. INTRODUCTION	3

1.1. Background	3

1.2. Objective of the mission	3

1.3. Conduct of the mission	4

2. EXECUTIVE SUMMARY	4

2.1. General	4

2.2. Specific comments	5

3. RECOMMENDATIONS TO THE IRANINAN COUNTERPART	5

4. RECOMMENDATIONS TO IAEA	7

REFERENCES:	8

APPENDIX I – MISSION PROGRAMME	9

APPENDIX II – LIST OF PARTICIPANTS	11

APPENDIX III - PRESENTATIONS	12

# 1. INTRODUCTION

## 1.1. Background

The BNPP was originally constructed with two 1300 MW PWRs by Siemens/KWU. The contract was cancelled in 1979. The Nuclear Power Plants Division (NPPD) of the Atomic Energy Organization of Iran (AEOI) finalized the turn-key contract with a main contractor from Russian Federation for complementing BNPP-1 with WWER-1000 reactor while utilizing the existing buildings, structures and equipment to the maximum extent feasible.

The Russian party was obliged to complete construction, perform the commissioning, physical start-up, start up to the rated power, testing, and trial operation. The responsibility for operation is in process of transferring to the Iranian party.

Regarding in-service inspection (ISI) activities, the counterpart Bushehr Nuclear Power Plant (BNPP) non-destructive testing (NDT) section continuously takes over the work from Russian party and responsibility as well, using the Russian technologies and standards. Their work in further safety operation of NPP is essential. Being able to conduct reliable ISI based on approved Program and to follow up worldwide trends in application of most advanced ultrasonic testing (UT) techniques and, for certain period of time, to retrieve risk-based inspection principles in their work, BNPP NDT section and Metal Testing Laboratory initiated a mission on this field. The mission had been realized between of BNPP and IAEA.

## 1.2. Objective of the mission

The objective of this mission was to conduct the presentations on applications of most advanced ultrasonic inspection method techniques such as phased array (PA) and time of flight diffraction (TOFD) i.e. their applicability at the WWER 1000 reactors.

In addition, objective of the mission was transferring knowledge to the Iranian party in regard to the newest ISI disciplines on UT field, with especial attention to PA and TOFD techniques, up-to date instruments, evaluation and flaw characterizations, gained experiences on the field. Also it is emphasized importance of inspection system qualifications based on European Network for Inspection Qualification (ENIQ) and IAEA methodologies as well as based on Russian standard РД ЭО 0487-05 “Типовые требования к порядку разработки технического задания, проведению истпытаний и условиям применения средств и методик эксплуатационного неразрушающего контроля на объектах использования атомной энергии”, 2005.

The Mission programme is attached in Appendix 1 of this Report.

The Mission programme is prepared to cover the following main topics:

- Overview of Advanced UT Techniques;

- UT Phased Array Technique: principles and usage, examples;

- UT TOFD Technique: principles and usage, examples;

- Overview of VVER 1200/1000 Main Components Inspection Systems using UT PA &TOFD Techniques;

- Inspection Systems Qualification Practice at Paks NPP/Hungary;

- Inspection Systems Qualification Practice at Kudankulam NPP/India (VVER 1000);

- ASME Code Adaptation, Transition from Russian Norms, In-service Inspections Ruling;

- QA aspects related to performance of PSI and ISI.

- The mission objectives were achieved and the duties between experts were discharged in accordance with main topics mentioned above. All activities were performed in close co-operation with the Counterpart’s representatives.

## 1.3. Conduct of the mission

- The IAEA mission has been conducted through lecture presentations by experts, Iranian counterpart presentations, and round table discussion.

- Versatile discussions are being held regarding the present on Site situation and in particular future strategy of Iranian counterpart based on best world practice taking into account a specific situation of Bushehr NPP.

- In addition, very active participation of Iranian experts has to be pointed out and their significant contribution in mission performance and effectiveness.

- The IAEA team consisted of two external experts who have extensive experience in the ISI/PSI activities of WWER-440/1000 Nuclear Power Plants and who are familiar with advanced approaches in nuclear industry worldwide.

- The List of Participants of the mission is provided in Appendix II of this Report.

# 2. EXECUTIVE SUMMARY

## 2.1. General

The operating organization has responsibility for ISI programme development and implementation in order to keep the safe and reliable operation of the NPP. In this concern the following items should be considered:

- The comprehensive programme for ISI of Bushehr NPP structure, systems and components (SSC’s) has to be implemented based on Russian codes and standards;

- The BNPP has prepared ISI programme plan. The scope of activities would be performed by own NDE personnel, Atomstroyexport experts and involvement of external ISI organizations;

- ISI shall be subject to Quality Assurance (QA) in relation to all aspect important to safety.

## 2.2. Specific comments

- Specific comments are given on the basis of the round table discussion and Iranian counterpart presentations:

- Needs to develop sophisticated ISI programme (computerized ISI data base);

- To perform a comprehensive ISI based on reference documentation taking into account the world practice, gained experience and Site specifics as well;

- To review all Contractor documentation in regard to effective performances of ISI. This includes review of the inspection procedures, inspection records to be produced, scope of inspection to be performed, documentation to be handed over to the BNPP, etc.;

  - A concern was expressed that there is no enough qualified NDT personnel on Iranian Counterpart in the field of nuclear industry. Also, the qualification and certification of BNPP NDT personnel is not quite apparent;

  - Needs that inspection systems (personnel, equipment, procedures) should meet any of the internationally recognized qualification approaches (ENIQ, IAEA Methodology for Qualification of ISI Systems for WWER NPPs, РД ЭО 0487-05) presently introduced in advanced ISI nuclear practice;

  - Due to fact that Bushehr NPP is first unit of Iran; Iranian Regulatory Body should build or designate correspondent rules to supervise and monitoring the BNPP PSI activities.

# 3. RECOMMENDATIONS TO THE IRANINAN COUNTERPART

Issue No. 1 - Recommendation for BNPP NDT section and Metal Testing Laboratory

In order to provide effective ISI programme implementation it is recommended to establish own core NDE group. An ISI concept approach should be based on a highly automated, remotely controlled equipment i.e. qualified inspection system (procedure, personnel and equipment).

Iranian strategy regarding ISI performances should be developed and may include the following options or their combinations:

  - To perform professional training of NDT persons through engagement of recognized professional companies, institutes or any other agency given those services. This professional training should include all experts involved in the performance of ISI activities at BNPP regardless are they Iranian citizens or foreign citizens i.e. experts engaged by BNPP to be a part of ISI group;

  - To establish own core qualified crew at the Site dealing with ISI;

  - To qualify and certify own non-destructive testing personnel according to internationally recognized standards or Russian standards as soon as possible. The on the job training is highly recommended through certain time period;

  - To consider subcontracting implementation of activities (whole or partially);

  - To purchase the necessary equipment on options selected.

Establishing of the NDT group would cover items as follows:

- Writing of working practice i.e. procedure for training, qualification and certification of the NDE personnel. The working practice would define requirements regarding training, experience and levels of qualification. The specific requirements as are vision test and education level requirements should be defined as well;

- NDE personnel selection based on knowledge and acquired experience in particular methods. Training, qualification and certification of NDE personnel shall be based on working practice requirements and each person should be certified in 2-3 methods;

- The training, qualification and certification of personnel should be performed in phases with respect to requirements of working practice and required level of qualification (for more details refer to ASNT SNT-TC-1A recommendations or ISO 9712:2012 - Qualification and certification of NDT personnel).

Issue No. 2 - Recommendation for NDE Equipment Provision

Based on ISI programme and on NDE group establishment as described in Issue No. 1 the following equipment is recommended to be provided for the training and for ISI performances as well:

- Ultrasonic testing:

| Item No. | Description | Quantity |
| --- | --- | --- |
| 1. | An advanced ultrasonic instruments for PA/TOFD techniques applications and A, B, C and D data presentation plus remotely operated scanners for inspection of reactor coolant piping (recommended) with versatile diameters and for the vessel shell examinations | 2 |
| 2. | UT PA and TOFD probes selected on the basis of ISI programme | 4 sets |
| 3. | Test specimens procurement and Calibration blocks (V1, V2 and other appropriate standard blocks with flat bottom holes) | 3 sets |

Note: aforesaid scope of equipment and specimens/calibration standards purchase is based on plan to ensure the equipment for the training of personnel (clean equipment) and the equipment for ISI performance (contaminated equipment) as well. It is recommended to focus on the piping welds inspection of the Du 850/350/300 piping (Main Coolant Piping, Surge Line and Safety injection systems).

Issue No. 3 – Other General Recommendations

Based on considerations presented in the paragraph 2 of this Report the following general recommendations are raised:

- To review the currently available design and as-built documentation and its systematic completion to provide all necessary data for effective ISI performance;

- To implement an appropriate on Site QA system;

- To review NDE procedures currently in force and propose changes in order to provide effective performance of ISI.

# 4. RECOMMENDATIONS TO IAEA

During IAEA mission some observations in regard to Iranian counterpart were noticed and therefore the following activities are recommended to IAEA:

- There is a need for additional assistance to Iranian counterpart through IAEA support which should include:

- Overall NDE professional training program development – establishing and implementation. Workshops or missions may not substitute formal training. Therefore it is strongly recommended to provide training in laboratory conditions (mockups, simulators, calibration standards, test specimens) to Iranian counterpart;

  - It is recommended establishing of the Pilot Project for, at least, one inspection system qualification (main coolant piping could be a good example). This Pilot project should include preparation of Technical Specification (Input information), Certification Procedure, Technical Justification and preparation of Working Procedure. This qualification approach is defined in the IAEA publication “Methodology for Qualification of In-service Inspection Systems for WWER Nuclear Power Plants”;

  - On the job training;

  - Equipment purchase (hardware and software).

- Recommendation for the follow up missions:

- The follow up missions should provide consistency in regard to recent mission programme and given recommendations.

# REFERENCES:

- Atomstroyexport: Typical pre-operational program of base material and welded joints of BNPP safety-related equipment and pipelines

- IAEA Safety Guide: Maintenance, Surveillance and In-Service Inspection in Nuclear Power Plants – Safety Standard Series No. NS-G-2.6

- РД ЭО 0487-05 “Типовые требования к порядку разработки технического задания, проведению истпытаний и условиям применения средств и методик эксплуатационного неразрушающего контроля на объектах использования атомной энергии”, 2005

- IAEA publication: Methodology for Qualification of In-service Inspection Systems for WWER Nuclear Power Plants

- European Network for Inspection Qualification: Qualification of Non-Destructive Testing – Report EUR 17299 EN

# APPENDIX I – MISSION PROGRAMME

| Date/Time | Lecture | Presenter | Presentation title |
| --- | --- | --- | --- |
| Saturday (06 July) |  |  |  |
| 9:00 – 09:30 | Introduction of Participants |  |  |
| 09:30- 10:30 | Overview of BNPP ISI Activities | Hossein Moammer Saeed Norouzi | Ultrasonic test by using automatic devices Familiarity with the method of performing UT test in BNPP |
| 10:30-11:30 | Overview of HRID Ltd. Activities | M. Cvitanovic | Introduction HRID Ltd. |
| 12:00-13:00 | Overview of Paks NPP NDT Department Activities | Gy. Paczolay | Introduction Paks |
| 14:00 – 15:00 | Overview of Advanced UT Techniques | M. Cvitanovic | PA & TOFD techniques |
| 15:15 – 16:00 | UT Phased Array Technique: principles and usage, examples | Gy. Paczolay | PA UT technique |
| Sunday (07 July) |  |  |  |
| 9:00 – 09:30 | Acquisition data evaluation considerations | Gy. Paczolay | Evaluation considerations |
| 09:30- 10:30 | UT TOFD Technique: principles and usage, examples | Gy. Paczolay | TOFD technique |
| 10:30-11:30 | Overview of VVER 1000 Main Components Inspection Systems using UT PA &TOFD Techniques | M. Cvitanovic | VVER 1000 Main Component Inspections using PA&TOFD techniques |
| 12:00-13:00 | Overview of VVER 1000 Main Components Inspection Systems using UT PA &TOFD Techniques-Cont. | M. Cvitanovic | VVER 1000 Main Component Inspections using PA&TOFD techniques |
| 14:00 – 15:00 | Inspection Systems Qualification Practice at Paks NPP/Hungary | Gy. Paczolay | Inspection qualification |
| 15:15 – 16:00 | Inspection Systems Qualification Practice at Kudankulam NPP/India | M. Cvitanovic | Inspection Systems Qualification |
| Monday (08 July) |  |  |  |
| 9:00 – 10:30 | ASME Code Adaptation, Transition from Russian Norms, In-service Inspections Ruling | Gy. Paczolay | ASME adaptation Ruling ISI at Paks NPP |
| 10:30- 11:30 | Overview of Du 850/350/300Welds and Elbows and SG Collector Weld UT PA Inspection Procedures-Assessments Criteria | M. Cvitanovic | Du 850/350/300 Welds and Elbows and SG Collector Weld UT PA Inspection |
| 12:00-12:30 | QA Aspects Related to performance of ISI Activities | M. Cvitanovic | ISI QA Aspects |
| 12:30-13:00 | Closing Session |  |  |

# APPENDIX II – LIST OF PARTICIPANTS

| No. | Name | Position | e-mail |
| --- | --- | --- | --- |
| 1. | Seyed Fazel Ghazi Ardekani | Head of metal laboratory Bushehr NPP | ghaziardakani@nppd.co.ir |
| 2. | Reza Kazemi Aghdam | Head of NDT of metal laboratory Bushehr NPP | rezalight2010@yahoo.com |
| 3. | Hossein Moammer | UT expert of metal laboratory Bushehr NPP | 58.hossein@gmail.com |
| 4. | Amir Movafeghi | Director General, Technical Support Department, Iran Nuclear Regulatory Authority | amovafeghi@aeoi.org.ir |
| 5. | Noushad Khajeh | NDT expert of metal laboratory Bushehr NPP | noush1353@mail.ru |
| 6. | Saeed Norouzi | Automatic control system expert of metal laboratory Bushehr NPP | noruzisaeed@ymail.com |
| 7. | Ali Haghighishad | Senior resident inspector of NNSD | 1976negar@gmail.com |
| 8. | Masoud Behzadinejad | Senior resident inspector of NNSD | m_Behzadi_75@yahoo.com |
| 9. | Mehrzad Vahidzadeh | Head of NDT OCE Ltd | M.V.Z.@yahoo.com |
| 10. | Vitali Tarasenko | Head of metal laboratory DATEX | tarasenko1970@mail.ru |
| 11. | Mohammad Asgharzadeh | DT expert of metal laboratory Bushehr NPP | asgharzadehMohammad@yahoo.com |
| 12. | Yunes Radi | DT expert of metal laboratory Bushehr NPP | yunes.radi@gmail.com |
| 13. | Vlademir Olynik | UT expert of metal laboratory DATEX |  |
| 14. | Uriy Kazlov | UT expert of metal laboratory DATEX |  |

IAEA experts:

| No. | Name | Position | e-mail |
| --- | --- | --- | --- |
| 1. | Mato Cvitanovic |  | mato.cvitanovic@hrid-ndt.hr |
| 2. | Gyozo Paczolay | NDT Engineer Paks NPP | paczolaygy@npp.hu |

# APPENDIX III - PRESENTATIONS

Note: The compact disc consisting of all presentations are integral part of this Report